Table of Contents

Abstract	1

1.	Introduction	2

Taguchi Approach	2

2.	Experimental Procedure	4

LATHE MACHINE:	4

Cutting Tool & Workpiece Material	5

Selection of Cutting Parameters:	6

Calculation of Material removal rate	7

PROCEDURE:	8

3.	RESULTS & DISCUSSION	10

Analysis of variance (ANOVA)	11

Main effect plots analysis	12

Mathematical modeling	15

4.	CONCLUSION	16

5.	References	17

### Abstract

In order to produce any product of desired quality by machining, proper selection of process parameters is very essential. This can be accomplished by Taguchi approach. The aim of our project is to investigate the effects of process parameters on surface finish and material removal rate (MRR) to obtain the optimal setting of these process parameters. Stainless Steel workpiece of 25mm diameter was used on a manual Lathe machine in the absence of any coolant. For the analysis purpose MINITAB was used as it is a very user friendly software to work on. All the results calculated are based on real time data obtained from actual hands on performance.

# Introduction

Surface roughness is mainly a result of process parameters such as tool geometry (i.e. nose radius, edge geometry, rake angle, etc.). and cutting conditions (feed rate, cutting speed, depth of cut, etc.). Surface roughness is harder to attain and track than physical dimensions are, because relatively many factors affect surface roughness. Some of these factors can be controlled and some cannot. Controllable process parameters include feed, cutting speed, tool geometry, and tool setup. Other factors, such as tool, work piece and machine vibration, tool wear and degradation, and work piece and tool material variability cannot be controlled as easily. The important cutting parameters discussed here are cutting speed, feed and depth of cut. It is found in most of the cases surface roughness decreases with increase in cutting speed and decrease in feed and depth of cut. Since these cutting parameters will decide about the type of chips which we expect at the time of machining of a single constant material thus we have to analyze them for no such built-up edge chips formation. The Taguchi method is statistical tool, adopted experimentally to investigate influence of surface roughness by cutting parameters such as speed, feed and depth of cut.

The aim of this experimental investigation is to evaluate the effects of the process parameters on mild steel work piece surface roughness and material removal rate by employing Taguchi’s orthogonal array design using manual lathe under dry environment. The mild steel is the most widely used grade among the other grades of steel. It is used for aerospace components and chemical processing equipment, for buildings, weapons, ships, trains etc.

# Taguchi Approach

Taguchi’s parametric design is an effective tool for robust design. It offers a simple and systematic qualitative optimal design at a relatively low cost. It has been widely used for the last two decades. The greatest advantage of this approach is to save the experimental time as well as the cost by finding out the significant factors. One of the important steps involved in Taguchi’s technique is selection of an orthogonal array (OA). An OA is a small set from all possibilities which helps to determine least no. of experiments, which will further help to conduct experiments to determine the optimum level for each process parameters and establish the relative importance of individual process parameters. To obtain optimum process parameters setting, Taguchi proposed a statistical measure of performance called signal to noise ratio (S/N ratio). This ratio considers both the mean and the variability. In addition to S/N ratio, ANOVA is used to indicate the influence of process parameters on performance measures. Taguchi proposed three categories of performance characteristics in the analysis of the S/N ratio, that is, the smaller the better, the higher the better, and the nominal the better. In the present work the first criterion selects the-smaller-the-better characteristic of the surface roughness and larger the better type for MRR.

Steps of Taguchi method are as follows:

1. Identification of main function, side effects and failure mode.

2. Identification of noise factor, testing condition and quality characteristics.

3. Identification of the main function to be optimized.

4. Identification the control factor and their levels.

5. Selection of orthogonal array and matrix experiment.

6. Conducting the matrix experiment.

7. Analyzing the data, prediction of the optimum level and performance.

8. Performing the verification experiment and planning the future action.

# Experimental Procedure

## LATHE MACHINE:

A lathe is a machine tool which is used to rotate a workpiece to perform various operations such as turning, facing, knurling, grooving etc., with the help of tools that are applied to the workpiece. The function of a lathe is to remove metal from a piece of work to give it a desired shape and size. In a lathe machine, the workpiece rotates against the tool. The tool is used to remove material from the workpiece. The direction of the motion of the tool is called a feed.

Figure 1: A Lathe Machine

## Cutting Tool & Workpiece Material

The cutting tool which provided with the lathe was a single point HSS cutting tool having the same tool angles as for a normal turning tool. The material of workpiece we used in our experiment was mild steel. Mild steel is a ferrous metal made from iron and carbon. It is a low-priced material with properties that are suitable for most general engineering applications. Low carbon mild steel has good magnetic properties due to its high iron content; it is therefore defined as being ‘ferromagnetic’.

Mild steel has a carbon content of between 0.16% and 0.29 % maximum with a relatively high melting point of between 1450°C to 1520°C. Steels with a higher carbon content than mild steel, have a lower melting temperature. This high melting temperature means that mild steel is more ductile when heated, making it particularly suitable for forging, cutting, drilling, welding and is easy to fabricate.

Table 1:Chemical Composition of Mild Steel.

| ELEMENT | WEIGHT % |
| --- | --- |
| C | 0.16 |
| Al | 0.07 |
| Si | 0.168 |
| Mn | 0.18 |
| P | 0.025 |
| Cu | 0.09 |
| Fe | Balance |

## Selection of Cutting Parameters:

Following cutting parameters were set to perform the whole experiment.

Table 2: Cutting Parameters

| Code | Cutting Parameter | Level 1 | Level 2 |
| --- | --- | --- | --- |
| A | RPM (rev/min) | 240 | 315 |
| B | FEED (mm/rev) | 0.16 | 0.22 |
| C | Depth of Cut (mm) | 0.1 | 0.2 |

The surface roughness of machined surface has been measured by a Surface Roughness Measuring instrument, self-contained instrument for the measurement of surface texture and is suitable for use in both the workshop and laboratory. Parameters available for surface texture evaluation are Ra. The parameters evaluations and other functions of the instrument are microprocessor based. The measurements results are displaced on an LCD screen and can be output to an optional printer or another computer for further results.

Table 3: L16 Mixed Level Design

| RPM | FEED | DOC | SURFACE ROUGHNESS (Ra) | MATERIAL REMOVAL RATE (mm3/min) |
| --- | --- | --- | --- | --- |
| 240 | 0.16 | 0.1 | 3.3 | 302.64 |
| 240 | 0.16 | 0.1 | 2.509 | 302.64 |
| 240 | 0.16 | 0.2 | 3.012 | 605.29 |
| 240 | 0.16 | 0.2 | 1.861 | 605.29 |
| 240 | 0.22 | 0.1 | 1.994 | 416.13 |
| 240 | 0.22 | 0.1 | 2.647 | 416.13 |
| 240 | 0.22 | 0.2 | 2.186 | 832.27 |
| 240 | 0.22 | 0.2 | 1.283 | 832.27 |
| 315 | 0.16 | 0.1 | 2.800 | 398.80 |
| 315 | 0.16 | 0.1 | 2.251 | 398.80 |
| 315 | 0.16 | 0.2 | 2.548 | 797.61 |
| 315 | 0.16 | 0.2 | 2.178 | 797.61 |
| 315 | 0.22 | 0.1 | 2.627 | 548.35 |
| 315 | 0.22 | 0.1 | 2.405 | 548.35 |
| 315 | 0.22 | 0.2 | 2.417 | 1096.71 |

## Calculation of Material removal rate

The material removal rate (MRR) was calculated using the following formula:

MRR = vfd

Where v= cutting speed, f= feed rate & d= depth of cut.

Cutting speed was calculated using following formula:

V=πDN

Where D= Df+2d and N= RPM

## PROCEDURE:

- Firstly open Minitab and click on new worksheet.

- From there go to STAT>DOE>TAGUCHI>CREATE TAGUCHI DESIGN and from there select the following.

Figure 2: Steps involved in Procedure

- Now Press OK and a new orthogonal array will appear having all the combinations of the factors in a L16 matrix.

- Simply perform the experiment with the required parameters and then fill the matrix accordingly.

- Calculate surface roughness using the surface roughness machine and also calculate the material removal rate using the above mentioned formula.

- After all the experimentation is finished return to the MINITAB window to further analyze the results.

- In MINTAB again go to STAT>DOE>TAGUCHI>ANALYZE TAGUCHI RESULTS.

Figure 3: Analysis of the Data

- From here we will select two responses i.e., surface roughness & material removal rate.

- For surface roughness we will select smaller the better from options and for material removal rate we will select larger the better.

- Now simply press ok to obtain the optimal results.

# RESULTS & DISCUSSION

Experiments are conducted to investigate the effects cutting parameters on the surface roughness and MRR of the mild steel work pieces. Table 3 gives experimental results. While estimating the mean and confidence interval, interaction effects are not considered.

## Analysis of variance (ANOVA)

The present study used ANOVA to determine the optimum combination of process parameters more accurately by investigating the relative importance of process parameters. Table 4 presents the results of ANOVA for surface roughness (Ra). It is observed from the ANOVA table, the feed (20.98 %) is the most significant parameter followed by rpm. However, the depth of cut has least effect in controlling the surface roughness. Statistically, F-test decides whether the parameters are significantly different. A larger F value shows the greater impact on the machining performance characteristics. Larger F-values are observed for feed as 2.677 and depth of cut. As seen from the ANOVA table 5, the influence of the depth of cut (95.01%) in affecting material removal rate (MRR) is significantly large. The rpm (4.99%) is the next significant factor. However, the feed has least effect in producing MR. Interaction effects are negligible for minimizing surface roughness and maximizing MRR.

Table 4:ANOVA Results for work piece surface roughness

| Source | DF | SS | MS | F | P |
| --- | --- | --- | --- | --- | --- |
| RPM | 1 | 0.1879 | 0.1879 | 0.398 | 0.593 |
| FEED | 2 | 0.9450 | 0.4725 | 2.677 | 0.183 |
| DOC | 4 | 0.7061 | 0.1765 | 0.633 | 0.653 |
| Error | 8 | 2.2298 | 0.2787 |  |  |

Table 5:ANOVA Results for MRR

| Source | DF | SS | MS | F | P |
| --- | --- | --- | --- | --- | --- |
| RPM | 1 | 20751.1228 | 20751.1228 | 0.700 | 0.491 |
| FEED | 2 | 59291.9646 | 29645.9823 | 0.265 | 0.779 |
| DOC | 4 | 446718.1938 | 111679.5485 | 2.834 | 0.098 |
| Error | 8 | 315273.5825 | 39409.1978 |  |  |

Figure 4:Main effect plot for Surface roughness

Main effect plots analysis

Figure 5: Results for Surface Roughness

The analysis is made with the help of a software package MINITAB 18. The main effect plots are shown in Fig.4 and Fig.6.These show the variation of individual response with the four parameters i.e. cutting speed, feed and depth of cut separately. In the plots, the x-axis indicates the value of each process parameter at two level and y-axis the response value. Horizontal line indicates the mean value of the response. The main effects plots are used to determine the optimal design conditions to obtain the optimum surface finish. Fig.4 shows the main effect plot for surface roughness. According to this main effect plot, the optimal conditions for minimum surface roughness are:

Figure 6: Main effect plot for MRR

According to main effect plot Fig. 6, the optimal conditions for maximum MRR are:

Figure 7: Results for MRR

## Mathematical modeling

A multiple linear regression model was developed for surface roughness and MRR using Minitab-14 software. The predictors are cutting speed, feed depth of cut and nose radius. The regression equation for surface roughness Ra = 2.760 + 0.217 RPM (rev/min) - 0.200 FEED (mm/rev)
- 0.218 Depth of Cut(mm)

Figure 8: Residual Plots for Surface Roughness

The regression equation for Material removal rate (MRR) = -156 + 72.0 RPM (rev/min) + 98.0 FEED (mm/rev) + 317.2 Depth of Cut(mm)

Figure 9: Residual Plots for Material Removal Rate

CONCLUSION

The experimental investigation conducted to turn mild steel using single point cutting tool made up of HSS at two levels by employing Taguchi technique to determine the optimal levels of process parameters. The ANOVA and F-test revealed that the depth of cut is the dominant parameter followed by rpm for surface roughness. In case of MRR response, the depth of cut is the dominant one followed by the rpm. The optimal combination of process parameters for minimum surface roughness is obtained at 315 rev/min rpm, 0.16 mm/rev feed, 0.1 mm depth of cut. The optimal combination of process parameters for maximum MRR is obtained at 315 rev/min rpm, 0.22 mm/rev feed, 0.2 mm depth of cut. It is observed that depth of cut plays an important role in both minimization of surface roughness and maximization of MRR. Whole experimental work was performed with hands on experience and results were then obtained after performing the experiment. All the results provided are considered to be accurate.

# References

- Al-Ahmari, A.M.A. Predictive machinability models for a selected hard material in turning operations. Journal of Material Processing Technology, 2007; 190:305-311.

- Aman Aggarwal, Hari singh, Pradeep kuma, Manmohan sing. Optimizing power consumption for CNC turned parts using response surface methodology and Taguchi’s Technique-A comparative analysis, Journal of Material. Processing Tech., 2008; 200: 373-384.

- https://www.metals4u.co.uk/blog/mild-steel-in-depth-guide

- Hari Singh and Kumar, P. Quality Optimization of Turned Parts (En 24 Steel) by Taguchi Method, Productivity journal, 2003; 44: 43-48.

- Ross, P.J. Taguchi Techniques for Quality Engineering, McGraw-Hill Book Company, 1996; New York.